Webb Research Corporation

82 Technology Park Drive, E. Falmouth, MA 02536-4441   (508) 548-2077  FAX (508) 540-1686

USER MANUAL – APEX-SBE PROFILER

APEX-SBE INSTRUMENTS

Serial numbers:  112 - 123

REV DATE: 02/23/00

IFM, Contract no. ZV-6369 RO, RO/CLV2000-15,

RO/CLV99-08

WRC Job no. 627

I.	ALKALINE BATTERY WARNING	2

II.	Reset and Self Test	2

III.	Mounting Damper PLATE	3

IV.	Deployment	4

V.    PROFILE DESCRIPTION…...…………………………………………………………………………………4

VI.	ARGOS DATA	6

A.	SERVICE ARGOS PARAMETERS	6

B.	DATA FORMAT	6

C.	TEST MESSAGE FORMAT	9

D.	CRC	10

E.	CONSTANTS	11

VII.	MISSIONS	12

VIII.	RECORDS & CALIBRATIONS	18

# ALKALINE BATTERY WARNING

The profiler contains alkaline "D" cells.

There is a small but finite possibility that batteries of alkaline cells will release a combustible gas mixture.  This gas release generally is not evident when batteries are exposed to the atmosphere, as the gases are dispersed and diluted to a safe level.  When the batteries are confined in a sealed instrument mechanism, the gases can accumulate and an explosion is possible.

Webb Research Corp. has added a catalyst inside of these instruments to recombine Hydrogen and Oxygen into H2O, and the instrument has been designed to relieve excessive internal pressure buildup by having the upper endcap release.

Webb Research Corp. knows of no way to completely eliminate this hazard.  The user is warned, and must accept and deal with this risk in order to use this instrument safely as so provided.  Personnel with knowledge and training to deal with this risk should seal or operate the instrument.  Webb Research Corp. disclaims liability for any consequences of combustion or explosion.

# Reset and Self Test

Profilers are shipped to the deployment site in Hibernate mode.  Shortly before deployment, reset the profiler by passing a magnet over the marked location on the pressure case.  The profiler will run a self-test, transmit for 6 hours with the bladder extended, and then begin its pre-programmed mission.

The six ARGOS transmissions during self-test and the transmissions during the initial 6 hour period contain data about the instrument and are outlined in  (V) ARGOS DATA, part (C) TEST MESSAGE FORMAT.

Procedure:

- Hold the provided magnet at RESET position marked on the hull for several seconds.

- Note:  The internal magnetic reed switch must be activated (held) for at least one second to reset the instrument.  (This is to provide a safety against accidental reset during transport.)  Thus, if the float does not respond as below, the instrument was probably not reset.

- The air pump will operate for 1 second.

- The PTT will transmit 6 times at 6 second intervals.  Place the ARGOS receiver/beeper close to the antenna to detect transmissions.

- The piston pump will begin to operate.  The piston will move to the retracted Ballast Position, if not already there, pause 2 seconds and then move to full extension.

- The bladder will expand, this should take 15 - 25 minutes.

- After the piston pump stops the air pump will come on and inflate the air portion of the bladder taking 20 - 30 seconds.

- The PTT will transmit at the mission specified ARGOS rate.

- 6 hours after reset, transmissions will cease, the piston pump will retract and the bladder will deflate, the profiler begins its programmed mission.

During self-test, the controller checks the internal vacuum sensor.  If the internal pressure has increased above a preset limit (i.e. hull leakage caused loss of vacuum), the instrument will not pump.  If you do not detect the 6 test transmissions, and if the bladder does not inflate, then the self-test has failed and the instrument should not be deployed.

# Mounting Damper Plate

To aid surface following, a damper plate is mounted to the outside of the pressure case.  The damper parts should be ballasted with each instrument, and numbered accordingly.

Below are instructions, which should be provided to the deployment crew:

- Remove the float from the crate and secure horizontally on foam cradles.

- Unpack the appropriate numbered stability plate.

- Remove the titanium hardware and one gray collar.

- Sandwich the clear plate between 2 gray collars.

- NOTE:  the chamfered edge of the collar must be toward the O-ring as shown below.

- Install 6 bolts firmly - use 2 flat washers for each bolt, one on each side of collar.

- Using two wrenches, tighten the second nut against the first to lock fasteners in place.

- Note:  Be sure to use the hardware particular to each instrument as each float is ballasted to .1 grams.

Place O-ring

into hull groove and between collars

Stability

Plate

Double-nut to

lock fastener

securely

# Deployment

- RESET instrument.

- SELF-TEST starts automatically (see above).

- When piston pump stops, air pump inflates, external bladder is full, PTT will transmit for 6 hours at ARGOS Repetition rate intervals.  Normally 90 seconds.

- Six hours after reset, the piston pump will retract and bladder will deflate.  Deploy before this time is up or reset the instrument again to re-initialize the 6 hour period.  The purpose is to have the instrument on the surface and receive test transmissions.

- Pass a rope through the hole in the damper plate.

- Holding both ends of the rope, carefully lower the float into the water.

- Take care not to damage the antenna.

- Do not leave the rope with the instrument, release one end and retrieve the rope.

- The float will remain on the surface until the 6 hour interval has expired.

# PROFILE DESCRIPTION

# A.	Park and Profile Feature

Terminology:

PARK:      intermediate depth at which the float drifts

PROFILE: maximum depth to which the float descends before profiling up.

DOWN time: spent during descent and at park depth.

UP time:  includes descent from park to profile depth, ascent, and time at surface

From Park to profile depth: approximately 6hrs

Ascent rate from Profile Depth: approximately .08 meters per second

Integer multiples of “trip interval” T are used to define Up time (U x T) and Down time (D x T).   See section VI, Missions.

Total Up time is typically set to 12 to 20 hours, increasing proportional to depth and amount of data to be transmitted per profile.  Another factor is deployment location:  due to the polar orbit of ARGOS, the number of passes per day increases at high latitudes.

UP time

DOWN time

DEPTH

Park

Surface

TIME

Profile

# B.	Sampling

APEX records a profile during ascent (ie upcast).  Bottom pressure may change due to several causes, such variation of insitu density, internal waves, float grounding in shallows, change of float mass, etc. APEX automatic depth adjustment will compensate in most, but not all, cases.

The number of sample points taken is proportional to depth, as per sample depth table below.

The first (i.e. deepest) sample is taken at the first point in the depth table above bottom pressure.

Depth Table No. 25

| Sample | Pressure | Sample | Pressure | Sample | Pressure |
| --- | --- | --- | --- | --- | --- |
| Point | (dbar) | Point | (dbar) | Point | (dbar) |
|  | Bottom |  |  |  |  |
| 1 | 1650 | 22 | 600 | 43 | 190 |
| 2 | 1600 | 23 | 550 | 44 | 180 |
| 3 | 1550 | 24 | 500 | 45 | 170 |
| 4 | 1500 | 25 | 480 | 46 | 160 |
| 5 | 1450 | 26 | 460 | 47 | 150 |
| 6 | 1400 | 27 | 440 | 48 | 140 |
| 7 | 1350 | 28 | 420 | 49 | 130 |
| 8 | 1300 | 29 | 400 | 50 | 120 |
| 9 | 1250 | 30 | 380 | 51 | 110 |
| 10 | 1200 | 31 | 360 | 52 | 100 |
| 11 | 1150 | 32 | 340 | 53 | 90 |
| 12 | 1100 | 33 | 320 | 54 | 80 |
| 13 | 1050 | 34 | 300 | 55 | 70 |
| 14 | 1000 | 35 | 280 | 56 | 60 |
| 15 | 950 | 36 | 260 | 57 | 50 |
| 16 | 900 | 37 | 250 | 58 | 40 |
| 17 | 850 | 38 | 240 | 59 | 30 |
| 18 | 800 | 39 | 230 | 60 | 20 |
| 19 | 750 | 40 | 220 | 61 | 10 |
| 20 | 700 | 41 | 210 | 62 | 4 or Surf |
| 21 | 650 | 42 | 200 |  |  |

*  The SeaBird CTD is not sampled at zero pressure, to avoid pumping the cell dry and/or

ingesting surface oil slicks.  The shallowest profile point is taken at either 4 dbar or at the last

recorded surface pressure plus 5 dbar, whichever value is larger.

# ARGOS DATA

## SERVICE ARGOS PARAMETERS

The user must specify various options to Service ARGOS.  These choices depend on how the user wishes to receive and process data.  Typical parameters are listed below:

- Standard location.

- Processing: Type A2 (pure binary input; hexadecimal output)

- Results Format: DS (all results from each satellite pass), Uncompressed.

- Distribution Strategy:  Scheduled, all results, every 24 hours.

- Number of bytes transmitted:  32 per message

Note:  Webb Research recommends all users to use ARGOS “Multi Satellite Service”, which provides receptions from 3 satellites instead of 2 for a small incremental cost.

## DATA FORMAT

Data is sent via ARGOS in 32 byte hex messages.  The number of 32 byte messages sent depends on the programmed quantity of temperature measurements per profile

Format for message number 1 only:

Byte #

- 01  CRC, described in section C.

- 02  Message number, Assigned sequentially to each 32 byte message (Total number of messages per profile is shown below).  Messages are transmitted in sequential order starting with 1 and incrementing by one for the data set.

- 03  Message block number, begins as 1 and increments by one for every ARGOS message 	data set.  This, combined with the ARGOS repetition rate (section VI), allows the user to 	track surface drift.  Byte 03 will roll-over at 256 and will reset to 1 on each new profile.

- 04 & 05  Serial number, identifies the controller board number.  (This may not be the same 	as instrument number.)

- 06  Profile number, begins with 1 and increases by one for every float ascent.

- 07  Profile length, is the number of six byte STD measurements in the profile.  Total 	number of bytes of STD data from each profile depends on the sampling strategy 	chosen.

- 08  Profile termination flag byte, can have the following values (hex):

00	Pressure reached surface pressure.

02	Pressure reached zero.

- Pressure unchanged for 25 minutes.  (Does not terminate profile.)

- Piston fully extended before surface

- UP time expired before surface and UP time was reset.

- 09  Piston position, recorded as the instrument reaches the surface.

- 10 & 11  Park temperature, sampled just before instrument descends to target depth.

- 12 & 13  Park salinity, sampled just before instrument descends to target depth.

- 14 & 15  Park pressure, sampled just before instrument descends to target depth.

- 16  Battery voltage, nominally at 15 volts and decreases throughout the life of the float.

- 17 & 18  Surface pressure, as recorded just before last descent with an offset of +5db.

- 19  Park vacuum, in counts,as recorded just before last descent.

- 20  Park piston position, the linear potentiometer count recorded at the park depth.

- 21  Profile piston position, the linear potentiometer count recorded at the target depth

- 22  Vacuum after bladder inflation, in counts, internal vacuum of instrument as recorded just after air  bladder has been inflated.

- 23 to 32  6 bytes in sequence:

2 bytes temperature

2 bytes salinity

2 bytes pressure

Format for message number 2 and higher:

Byte #

- 01  CRC, described in section C.

- 02  Message number

- 03 to 32  6 bytes in sequence:

2 bytes pressure

2 bytes temperature

2 bytes salinity

Message Format and Sampling Depths

| BTYE # | MSG 1 | BTYE # | MSG 2 | MSG 3 | MSG 4 | MSG 5 | MSG 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 & 11 | Tp | 3 & 4 | P2 | P7 | P12 | P17 | P22 |
| 12 & 13 | Sp | 5 & 6 | T3 | T8 | T13 | T18 | T23 |
| 14 & 15 | Pp | 7 & 8 | S3 | S8 | S13 | S18 | S23 |
| 17 & 18 | Ps | 9 & 10 | P3 | P8 | P13 | P18 | P23 |
| 23 & 24 | T1 | 11 & 12 | T4 | T9 | T14 | T19 | T24 |
| 25 & 26 | S1 | 13 & 14 | S4 | S9 | S14 | S19 | S24 |
| 27 & 28 | P1 | 15 & 16 | P4 | P9 | P14 | P19 | P24 |
| 29 & 30 | T2 | 17 & 18 | T5 | T10 | T15 | T20 | T25 |
| 31 & 32 | S2 | 19 & 20 | S5 | S10 | S15 | S20 | S25 |
|  |  | 21 & 22 | P5 | P10 | P15 | P20 | P25 |
|  |  | 23 & 24 | T6 | T11 | T16 | T21 | T26 |
|  |  | 25 & 26 | S6 | S11 | S16 | S21 | S26 |
|  |  | 27 & 28 | P6 | P11 | P16 | P21 | P26 |
|  |  | 29 & 30 | T7 | T12 | T17 | T22 | T27 |
|  |  | 31 & 32 | S7 | S12 | S17 | S22 | S27 |

| BTYE # | MSG 7 | MSG 8 | MSG 9 | MSG 10 | MSG 11 | MSG 12 | MSG 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 & 4 | P27 | P32 | P37 | P42 | P47 | P52 | P57 |
| 5 & 6 | T28 | T33 | T38 | T43 | T48 | T53 | T58 |
| 7 & 8 | S28 | S33 | S38 | S43 | S48 | S53 | S58 |
| 9 & 10 | P28 | P33 | P38 | P43 | P48 | P53 | P58 |
| 11 & 12 | T29 | T34 | T39 | T44 | T49 | T54 | T59 |
| 13 & 14 | S29 | S34 | S39 | S44 | S49 | S54 | S59 |
| 15 & 16 | P29 | P34 | P39 | P44 | P49 | P54 | P59 |
| 17 & 18 | T30 | T35 | T40 | T45 | T50 | T55 | XX |
| 19 & 20 | S30 | S35 | S40 | S45 | S50 | S55 | XX |
| 21 & 22 | P30 | P35 | P40 | P45 | P50 | P55 | XX |
| 23 & 24 | T31 | T36 | T41 | T46 | T51 | T56 | XX |
| 25 & 26 | S31 | S36 | S41 | S46 | S51 | S56 | XX |
| 27 & 28 | P31 | P36 | P41 | P46 | P51 | P56 | XX |
| 29 & 30 | T32 | T37 | T42 | T47 | T52 | T57 | XX |
| 31 & 32 | S32 | S37 | S42 | S47 | S52 | S57 | XX |

Tp, Sp, and Pp are park Temperature, Salinity, and Pressure values

Ps is surface Pressure

T, S, and P are Temperature, Salinity, and Pressure values

XX is an invalid data point

Data format chart above  assumes that bottom pressure (maximum hydrostatic pressure at start of profile) is 1500dbar.  Data format will change if bottom pressure varies.

## TEST MESSAGE FORMAT

The test message is sent whenever an I2 command is given, the six transmissions during the startup cycle, and during the six hour surface mode period prior to the first dive.  Each test message has 32 bytes, in hex unless otherwise noted, with the following format:

Byte #

- 01  CRC, described in section C.

- 02 Message block number, begins as 1 and increments by one for every ARGOS message.

- 03 & 04 Serial number, identifies the controller board number.  (This may not be the same    as instrument number.)

- 05  Hour, the following is the time from startup (in decimal)

- 06 Minutes

- 07 Seconds

- 08  Flag (2) *

- 09 & 10  Current pressure

- 11  Battery voltage

- 12  Internal vacuum, in counts

- 13  Piston position

- 14  Flag (1) **

- 15  ARGOS Repetition rate Constant, (n + 1) * 2 = seconds

- 16  Up time

- 17 & 18  Down time

- 19  Interval time

- 20 & 21  Park pressure

- 22  Park piston position

- 23  Depth correction factor

- 24  Ballast piston position

- 25  Fully extended piston position

- 26  OK vacuum count at launch

- 27 & 28  Profile pressure

- 29  Initial piston extension, in counts.

- 30  Month, software version number (in decimal).

- 31  Day, software version number (in decimal).

- 32  Year, software version number (in decimal).

* Flag (2)  byte:  1  Unused                                          **Flag (1)  byte:  1  Trip interval time

2  Pressure reached zero                                                2  Profile in progress

3  25 minute Next Pressure timeout                              3  Timer done

4  fully extended before surface                                    4  UP/ DOWN

5  Up time out before surface                                        5  Data entry error

6  Test message at turn on                                             6  Piston fully extended

7  Six hour surface message                                           7  Piston motor running

8  Arithmetic round up                                                  8  Negative SBE number

## CRC

Because ARGOS data may contain transmission errors, the first byte of each message contains an error checking value.  This value is a Cyclic Redundancy Check (CRC), and is calculated as a function of the message content (bytes 2 to 32).

- For each message, calculate a CRC value

- Compare the calculated CRC to the transmitted CRC (byte no. 1)

- If the calculated and transmitted CRC values are not equal, the message has been corrupted and should be deleted before further data processing.

Below is a sample program (in BASIC) to calculate the CRC value for a message.  This program can be provided upon request in Basic, Fortran or C.

DECLARE FUNCTION CRC% (IN() AS INTEGER, N AS INTEGER)

‘CRC routine to check data validity in ARGOS message.

‘Bathy Systems, Inc. RAFOS Float data transmission.

‘3 December, 1990.

‘The 1st of 32 bytes in an ARGOS message is the CRC.

‘The function CRC will compute CRC for byte 2 through 32.

‘Hasard is used for Random because Random is reserved by BASIC.

‘Stored as file CRC in C:\RAFOS\RAF11.

DECLARE SUB Hasard (ByteN AS INTEGER)

DEFINT A-Z

DIM in(32) AS INTEGER

‘RAF11F message number 08 HEX ID 11502 01-02-93   CRC is O.K.

A$ = “8F00081C8E47239148A4D2E9743A1D0E070381C06030984C2693492492C964B2”

N = 32

FOR I = 1 to N

in(I) = VAL(“&H” + MID$(A$, 2 + I - 1, 2))

NEXT I

PRINT in(1); CRC(in(), N);

FUNCTION CRC% (IN() AS INTEGER, N AS INTEGER) STATIC

DIM ByteN as INTEGER

I = 2

ByteN = in(2)

DO

CALL Hasard(ByteN)

I = I + 1

ByteN = ByteN XOR in(I)

LOOP UNTIL  I = N

CALL Hasard (ByteN)

CRC = ByteN

END FUNCTION

DEFINT A-Z

SUB Hasard (ByteN AS INTEGER) STATIC

x% = 0

IF ByteN = 0 THEN ByteN = 127: EXIT SUB

IF (ByteN AND 1) = 1 THEN x% = x% + 1

IF (ByteN AND 4) = 4 THEN x% = x% + 1

IF (ByteN AND 8) = 8 THEN x% = x% + 1

IF (ByteN and 16) = 16 THEN x% = x% + 1

IF (X% AND 1) = 1 THEN

ByteN = INT(ByteN / 2) + 128

ELSE

ByteN = INT(ByteN / 2)

END IF

END SUB

## CONSTANTS

The pressure is measured every 6 seconds.  Temperature, salinity and pressure are measured and stored at each point in the depth table.

Two hex bytes are stored for each sensor.  The decimal numbers from the STD sensors are converted to hex for compression in the ARGOS transmission as follows:

Temperature:	first 5 digits, 1 milli-degree resolution.

Salinity:  	5 digits starting with the second digit (first digit is usually a 3).

Pressure:  	first 5 digits, 10 cm resolution.

To convert the hex ARGOS message back to decimal numbers:

hex 	 	dec	=	converted	units

Temperature:	23CA		9162	=	9.162 		C

Salinity:	see below

Pressure:	1D4C		7500	=	750.0 		decibars

Note:  Several Salinity examples are given due to the fact that the hex range is 0000 to FFFF and the first salinity digit in the ocean is usually a 3 but can be otherwise.  Some knowledge of the area of ocean being profiled is necessary to convert the compressed data.

hex 	 	dec	=	converted

0001 		00001	=	30.0001

FFFF 		65535 	= 	36.5535 (after FFFF hex the numbers will roll over)

10001 		65537	=	36.5537 (add a 1 to the front of the rolled over hex value)

153B		05435	=	30.5435 (could be this salinity or…)

153B		05435	=	20.5435 (could be this salinity or…)

1153B		70971 =	37.0971 (could be this salinity if the numbers rolled over)

Care to inspect a graph of the converted salinities with regard to depth and temperature must be exercised in order to quality control the conversions of compressed hex to converted salinity.

Voltage (V)  =  counts/10 + .6  (counts is in decimal number) nominally 15 V and decreasing.

Vacuum (inHg)  =  counts *-0.376 + 29.15  (counts is in decimal number) nominally 5 inHg.

# MISSIONS

INSTRUMENT #112

APEX version  01 27 00  sn 0369

A0A5E ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

095  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

068  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #113

APEX version  01 27 00  sn 0370

A0AAB ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

094  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

070  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #114

APEX version  01 27 00  sn 0371

A0AF8 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

096  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

069  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #115

APEX version  01 27 00  sn 0372

A0B12 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

095  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

070  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #116

APEX version  01 27 00  sn 0373

A0B41 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

094  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

072  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #117

APEX version  01 27 00  sn 0374

A0BB4 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0200  d-bar park pressure.    P1

095  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

070  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #118

APEX version  01 27 00  sn 0375

A0BE7 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

086  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

070  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #119

APEX version  01 27 00  sn 0376

A0C1C ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

084  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

069  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #120

APEX version  01 27 00  sn 0377

A0C4F ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

085  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

069  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #121

APEX version  01 27 00  sn 0378

A0CBA ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

084  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

070  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #122

APEX version  01 27 00  sn 0379

FE53B ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

082  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

069  OK vacuum count.         P8

025  Initial piston extention.

INSTRUMENT #123

APEX version  01 27 00  sn 0380

FE568 ARGOS ID number.

044  seconds repetition rate.

001  hour Trip interval.

218  intervals DOWN.

022  intervals UP.

0400  d-bar park pressure.    P1

084  park piston position.    P2

014  ascent rate correction.  P3

022  ballast piston position. P4

250  piston full extention.   P5

1500  d-bar profile pressure. P6

030  profile piston position. P7

069  OK vacuum count.         P8

025  Initial piston extention.

# RECORDS & CALIBRATIONS